VGAIMAGE    Version 1.2   11/12/89

By: Hoke Johnson

VGAIMAGE is a shareware image viewer/maker program for the IBM PC.  It is

optimized for use with VGA cards and monitors but allows viewing of Gif images

on EGA monitors but currently is really only good for 16 color EGA Gif

pictures.  VgaImage was created to provide many additional image viewing

features which I haven't been able to find in other image viewing programs.

This is the third release of VGAIMAGE and it currently supports GIF and TIFF

file formats.

NEW FEATURES:

VGA 360x480x256 mode support for standard VGA

VGA 320x400x256 mode support for standard VGA

Virtual Screen Disk Buffer to process files larger than VGA screen.

50 percent reduction works on more modes.

FEATURES:

Gif file viewing and processing

Viewing and processing of 2, 16 and 256 level B&W Tiff files

Conversion of Halftoned and 16 level Tiff files to 64 level Gif files

Conversion of 256 level Tiff files to 256 level Gif files

Support for Paradise VGA boards in 640x400x256 mode

High Resolution VGA Support for Tseng Labs VGA boards (Orchid Designer VGA)

and Paradise VGA boards

Zoom mode support for Tseng Labs VGA boards

360x480x256 and 320x400x256 mode support for standard VGA boards

16 level Gray Scale display of Gif files on normal VGA boards at 640x480

16 level Gray Scale display of Gif files on Tseng Labs boards at 1024x768

50% size reduction on large Gif files to fit them on the screen and ability

to write them as reduced size Gif files

Gif and Tiff header display

Brightness controls

Smooth Vertical scrolling of large images

Setting of the number of bits of resolution to be used for the RGB primaries

(Between 1 and 6 bits of resolution)

STARTUP PARAMETERS:

Usage: vgaimage [GCType]

Where GCType is: one of the following graphic card types:

EGA, SUPEREGA, VGA, DESIGNERVGA, PARADVGA

Graphic card type can also be changed once the program is running by pressing

the "D" key.

HARDWARE REQUIREMENTS:

IBM PC with at least 256K memory and a VGA or EGA graphics card and suitable

monitor.  The graphics card should also have at least 256K of memory.  A

hard disk is not required but is recommended if the Virtual Disk Buffer is

used since it performs a large number of I/O,  a large RAM disk works even

better.

This program works well with standard VGA cards,  VGA cards based on the

Tseng labs chip set (such as Orchid Designer VGA cards etc) and on Paradise

VGA boards.  With the standard VGA card, resolutions of 320x200x256,

320x400x256, 360x480x256 and 640x480x16 shades of gray are possible. With

the Tseng labs based cards all the modes of the standard VGA card are

possible plus 640x480x256, 800x600x256 and 1024x768x16 shades of gray.  With

the Paradise VGA cards all the modes of the standard VGA card are possible

plus 640x400x256 colors.  The extended modes of the Tseng labs based cards

(800x600x256 and 1024x768x16) require multi-sync monitors.

USING VGAIMAGE

Once VGAIMAGE is started, a list of available files for viewing is displayed

in the file section which is the upper 2/3 of the screen and a control panel

is displayed in the control section which is the lower 1/3 of the screen.

The file section is used to select files for viewing or conversion and the

control section is used to control the way files are displayed and converted.

The file selection contains a list of available files that match the input

file type.  File types currently supported are Tiff and Gif,  Tiff files must

have an extension of .TIF and Gif files must have an extension of .GIF .

Change the highlighted file by using the cursor arrow keys as well as the

home and end keys.  With desired file selected press the enter key to view

the selected file,  when done viewing press the enter key again to return to

file selection mode.  Currently the first 120 files in the current directory

are available for viewing.

The control section shows the current state of the display control variables.

Each variable is preceded by a description and an upper case letter in

parentheses.  The state of the variable is changed by depressing the letter

in parentheses,  either upper or lower case.  For example:

Display Type   (D): VGA

This example shows that the current display type is VGA and to change to

another display type,  press the "D" key to sequence through all possible

display types.

The available control variables and other keys that do things are described

below:

F1 - Help Screen

I  - Changes the input file type for viewing.  Currently GIF and TIFF are

supported.  Files must have extensions .GIF and .TIF respectively.

O  - Changes the output file type to be written during a write command.

Currently only GIF is supported.

F9,\ Change directory.  Press F9 or \ and then respond with the desired new

directory followed by a Carriage Return.

D  - Change the display type.  Pressing the "D" key sequences through all

the currently supported Graphic Card types.

A  - Aspect adjustment flag on/off. The aspect adjustment flag is normally

on and affects the display when the display type is set to standard

VGA and the 360x480x256 mode is being used.  The result is that every

other pixel in the horizontal dimension is discarded.  Normal aspect

GIF files such as 640x480 images should be viewed with this flag on,

there are some GIF images that have a 360x480 size with the aspect

already adjusted, these should be viewed with the aspect flag off.

H  - Causes the file header contents to be displayed prior to the image

being displayed.  Header remains displayed until an "enter" or CR is

pressed.

L  - Causes the display program to shrink the displayed image by 50% in both

the horizontal and vertical directions.  This mode now works in VGA

320x200 mode as well as some of the super VGA modes such as 640x480,

800x600 and 640x400.  It doesn't work in 360x480x256 mode.

E  - Changes to a 16 level gray scale display mode for Gif files.  For

normal IBM VGA type boards this gives a 640x480 display with 16 levels

of gray.  With Designer VGA boards (Tseng Labs) it is possible to

switch between 640x480 and 1024x768 modes at 16 levels of gray by using

the F8 key.

F8 - High Resolution Lock mode.  This is only usable with the Designer VGA

type boards or boards with Tseng Labs chips.  This gives 800x600 and

256 colors with the VGA 16 color B&W mode disabled (E key) and

1024x768 and 16 levels of gray with the VGA 16 color B&W mode enabled.

(A multisync monitor is also required)

F10 or ESC are used to exit from VGAIMAGE.

S  - Sets internal special flag which allows a modified Orchid Designer VGA

board to display 800x600x256 mode properly.  Normally this setting

should be left alone (ie OFF), if anyone is interested, write me for

details.

G  - In Gif view mode this key selects the number of bits of resolution for

each primary color to use when viewing.  In Tiff view mode this key is

used to select the number of levels of gray to use when the Tiff file

is displayed.  This value can be 2, 16, 64 or 256.  Not all Tiff files

can be viewed in all possible levels of gray output.

The supported Tiff renderings are:

2 level Tiff to   2 level display

2 level Tiff to  64 level display

16 level Tiff to  16 level display

16 level Tiff to  64 level display

256 level Tiff to 256 level display

V  - Virtual Disc Buffer Enable/Disable.  When the Virtual Disc Buffer is

enabled,  the bytes of each scan line as they are read from either a GIF

or a Tiff file are written into a disc file VGAIMAGE.VRT.  This allows

images larger than the available screen memory to be processed.

COMMANDS WHILE IMAGE IS BEING VIEWED

B  - Changes image to BW equivalent picture

C  - Restores original color picture

T  - Decreases the number of bits of resolution for each primary. Values

supported are 1 to 6 bits of resolution.  This can be used to get a

rough idea on the amount of resolution that a given image has by

starting out at the maximum and reducing until you can see a

deterioration in the image quality.

G  - Complement to the T command,  increases the number of bits of

resolution for each primary.

+  - Increase the brightness of the displayed image.

(-) - Decrease the brightness of the displayed image.

0  - Restore image to its original brightness (Zero key)

W  - Writes the currently displayed image in the specified outfile format.

Currently only GIF is supported.  The actual image can be longer than

the display screen,  for instance 640x819 for the Designer VGA card.

Large files can be output in GIF format in a minute or two on an AT

machine.  The output filename of the new file is the same as the first

filename with the new extension if we are converting from one file type

to another.  If the intended new filename already exists,  a new

filename is used which has the last letter of the filename

"incremented".

If the Virtual Disc Buffer is not enabled,  VGAIMAGE reads the image

from the display memory and creates the output file,  this works for

VGA 320x200x256, DESIGNERVGA 320x200x256, 640x350x256, 640x480x256,

800x600x256 modes.  If "W" is pressed without Virtual Disc Buffer

enabled and a display mode other than those just mentioned is in use,

VGAIMAGE terminates the display of the current image and returns to

the file selection menu.

If the Virtual Disc Buffer is enabled,  VGAIMAGE reads the image from

the Virtual Disc Buffer file VGAIMAGE.VRT and creates the output file.

The Virtual Disc Buffer only functions when the display type is one of

VGA, DESIGNERVGA or PARADVGA.  Images larger than the display can be

handled with the Virtual Disc Buffer.

Currently you can tell that the GIF write process is in progress by

the disk busy light,  the completion of the write process is signalled

by two short beeps.

Z  - Turns Zoom mode on and off for Tseng boards (Designer VGA)

<  - Decrease Zoom factor (actually the "," key)

>  - Increase Zoom factor (actually the "." key)

While in Zoom mode you can scroll around the picture by using the

Home, PGup, PGdn, and cursor arrow keys.

M  - Smooth vertical scrolling of images which are larger than the display

screen.  This is usable on Designer VGA boards and VGA boards with

images that have a height greater than 480 lines.

NOTES ON USING THE VIRTUAL DISC BUFFER

The Virtual Disc Buffer is a disc based copy of the pixels which make up

an image.  It allows for images larger than the display memory to be

converted or processed.  It also allows for images which VGAIMAGE can't

readback from display memory to be converted or processed.  The virtual disc

buffer should only be enabled when you want to output a GIF file such as when

you are converting from Tiff to GIF or when you are adjusting the brightness

on a GIF file or doing a 50% reduction on a GIF file.  If the virtual disk

buffer is enabled all the time,  VGAIMAGE will operate much slower due to

the great increase in extra I/O.  A file VGAIMAGE.VRT is created in the

current directory and used as the virtual disc buffer,  upon exit of

VGAIMAGE, the file VGAIMAGE.VRT is left and is not deleted,  if space is a

problem, be sure to delete the VGAIMAGE.VRT files.  The virtual disk buffer

shouldn't be with a floppy drive unless you are prepared to wait a very

long time since the virtual disc buffer requires a lot of I/O.  For best

results use with a large Ram Disc or a fast hard disk.

VGAIMAGE can read interlaced images but normally only outputs non-interlaced

images.  The one exception is for a GIF image which is already interlaced and

is displayed with the virtual disk buffer enabled,  a subsequent write of that

image will be as an interlaced image.

NOTES ON VIEWING TIFF FILES AND MAKING GIF FILES FROM TIFF FILES

Black and White Tiff files can be viewed and converted into Gif files.  Tiff

file support is currently only available for VGA type boards.  Tiff files

with 1 bit per pixel, 4 bits per pixel and 8 bits per pixel are supported.

When viewing Tiff files with 1 bit per pixel, and Tiff Gray Levels set to

64, VGAIMAGE converts to an image with 64 levels of gray.  In doing so the

pixel width and height of the original Tiff Image is reduced by a factor of

eight.  For instance a 1 bit per pixel Tiff image with a width of 4800

pixels and a height of 6400 pixels will reduce to an image with a width of

600 pixels by a height of 800 pixels where each pixel has six bits per

pixel.  It is also possible to view Tiff images without the conversion to 6

bits per pixel by setting the Tiff Gray Levels to 2 by pressing the G key.

Currently it is not possible to make a valid Gif file when there is only one

bit per pixel.

When viewing Tiff files with 4 bits per pixel, VGAIMAGE can display an image

with either 16 or 64 levels of gray.  The Tiff Gray Levels setting controls

which mode is used (G key).  When viewing Tiff files with 4 bits per pixel

and 64 levels of gray displayed, the height and width of the image is

reduced by a factor of 2, thus a 1200 x 960 image will reduce to 600 x 480.

VGAIMAGE has been tested with a few 256 level Tiff files,  this type of Tiff

file should produce the best results.  The 6 bit per primary limitation of

the VGA card causes the displayed image to be of only 64 levels but the full

resolution is maintained when creating an output GIF file.  (If the 256

level Tiff file is brightened or darkened prior to saving as a GIF then the

gray scale levels will be rounded off to 6 bits or 64 gray levels in the

palette)

VGAIMAGE has been tested with Tiff files created by HP Scanning Gallery and

with Tiff files created by HALO DPE.  Good image results are achieved by

using HP Scanning Gallery,  using the Fatting Dither setting and a custom

resolution of 600 dots per inch.  Experiment with setting the brightness

control to "Lighten" and "Normal" to find the best result.  This allows a

normal A size sheet of paper to be scanned and produces an image of

approximately 600 x 800 with 64 levels of gray.  Normally it is necessary to

brighten the image by pressing the "+" key approximately 6 to 10 times prior

to saving the image as a Gif file.  The Tiff files produced using this

method vary from 2.5 to 4.4 Megabytes but can be compressed using one of the

available "archiving" utilities to produce files < 1.2 Mbytes which allows

them to be stored on 1.2 M floppies and thus moved between machines.  Images

can be produced using the 16 levels of gray however I have found that the

images are usually darker and need to be brightened so much that you loose

image quality.  The 16 levels of gray would probably do well on a picture

that was very bright to start with.  Images that are dark to start with

generally don't scan well.

SHAREWARE:

VGAIMAGE is user supported software.  VGAIMAGE is made available for a

free ten day trial period.  If you find VGAIMAGE useful,  you may license

the program for non-commercial use on one machine by sending $15.00 to:

Hoke Johnson

P.O. Box 35602

Monte Sereno, CA 95030

All other rights reserved.

VGAIMAGE may not be modified in any way by anyone other than the author.

VGAIMAGE may be freely distributed to others only in its complete and

unmodified form with no fee charged other than postage, media or

duplication charges.

Regardless of how your copy of VGAIMAGE was obtained,  it is requested

that you comply with the above licensing agreement.

All warranties are disclaimed,  including damage to hardware and/or

software from use of this product.

FUTURE ENHANCEMENTS

The following are some of the planned future enhancements:

-Support for more types of high resolution VGA boards

-Color GIF file creation from scanned Tiff files

-Pseudo 640x400x256 color mode on standard VGA

-Better error checking during GIF decoding

-PCX file format support

-Support for more types of TIFF files (64 gray level)

-Test escape (ESC) key during image decoding or writing to abort current

action.

FEEDBACK AND COMMENTS

I welcome your comments, you can contact me by mail

Hoke Johnson

P.O. Box 35602

Monte Sereno, CA 95030

For now,  I do not plan on trying to support compressed Tiff files since the

packages I have seen have the option of specifying whether the output Tiff

file is compressed or not and the compression efficiency is not that good

anyway.

KNOWN BUGS, FEATURES OR LIMITATIONS:

- When using virtual disk buffer, a file VGAIMAGE.VRT is created in the

currently selected directory,  this file is not deleted when VGAIMAGE

terminates and thus the temporary file VGAIMAGE.VRT is left undeleted.

This file contains a sequence of bytes which are the values of the pixels

for the last image which was decoded while the virtual disk buffer was

enabled.  A source for the uncompressed bytes of a Gif image.

- Empty Gif files are created in the current directory.  This happens when

trying to write a Gif file and the Virt Disk Buffer is not enabled and the

display mode is one which cannot be read.  For example displaying in

Paradise 640x400 mode with the Virt Disk Buffer disabled.

- Some modes don't vertically scroll or pageup and pagedown smoothly and

tend to cause the picture to jump sideways, this happens on the PARADVGA

mode.

- When using the 50% reduction and vertically moving the picture with either

the pageup/pagedown keys or the "m" key, the scrolling continues on beyond

the bottom of the picture and sometimes the top of the picture will start

to appear at the bottom only shifted sideways.  This also happens on the

1024x768x16 B&W mode on the Designer VGA.

- 2 level Tiff files can be viewed but cannot be saved correctly as GIF files.

CHANGES:

VGAIMAGE Version 1.0,  03/27/89 - Initial Release

VGAIMAGE Version 1.1,  05/15/89

- Fixed problem which didn't sort directory listing properly when there

were only two files to be displayed

- Enhanced GIF output routine to produce maximally compressed files and

improved speed drastically by using hashing and improving the disk

i/o routines.

- Added support for displaying 640x400x256 colors on the Paradise VGA

board.

- Decreased the amount of memory required for displaying very large GIF

files.

- Added additional Tiff Tag handling and allowed program to attempt to

display image even if there are some unhandled tags.  A list of

unhandled tags is also listed when the Header display is enabled.

- Added support for 256 level Tiff files

VGAIMAGE Version 1.2,  11/12/89

- Allow files to be opened which are secured as read-only

- Fixed error handling of opening invalid Tiff files

- Added Virtual Screen Disc Buffer support which allows image conversion

on images which exceed the size of the display memory.

- Added support for 360x480x256 and 320x400x256 modes on the standard VGA

card.  For the 360x480x256 mode there is an aspect adjustment flag which

allows 640x480x256 images to display.

- Added line clipping when displaying images larger than the screen.

- Added support for 50 percent reduction on super VGA modes such as

640x480x256, 800x600x256 and 640x400x256.