SuperProbe(1)					SuperProbe(1)

NAME

SuperProbe  - probe for and identify installed video hard-

ware.

SYNOPSIS

SuperProbe [-verbose] [-no16] [-excl list] [-mask10]

[-order list] [-noprobe list] [-bios base]

[-no_bios] [-no_dac] [-no_mem] [-info]

DESCRIPTION

SuperProbe is a a program that will attempt  to	determine

the  type  of video hardware installed in an EISA/ISA/VLB-

bus system by checking for known registers in various com-

binations  at  various  locations  (MicroChannel	and  PCI

machines may not be fully supported; many  work	with  the

use  of the -no_bios option).  This is an error-prone pro-

cess, especially on Unix (which usually	has  a	lot  more

esoteric	hardware  installed  than  MS-DOS system do), so

SuperProbe may likely need help from the user.

SuperProbe     runs     on     SVR3,	SVR4,	Linux,

386BSD/FreeBSD/NetBSD,  Minix-386, and Mach.  It should be

trivial to extend it to work on any other Unix-like  oper-

ating system, and even non-Unix operating systems.  All of

the OS dependencies are isolated to a single file for each

OS.

At  this time, SuperProbe can identify MDA, Hercules, CGA,

MCGA, EGA, VGA, and an entire horde of SVGA chipsets  (see

the  -info  option,  below).  It can also identify several

HiColor/True-color RAMDACs in use on SVGA boards, and  the

amount  of video memory installed (for many chipsets).  It

can identify 8514/A and some derivatives, but not XGA,  or

PGC  (although  the  author intends to add those capabili-

ties).  Nor can it identify other esoteric video	hardware

(like Targa, TIGA, or Microfield boards).

OPTIONS

-verbose

SuperProbe  will	be  verbose  and provide lots of

information as it does its work.

-no16   SuperProbe will not attempt to use any ports  that

require 16-bit I/O address decoding.  The original

ISA bus only specified that I/O ports  be  decoded

to  10  bits.  Therefore some old cards (including

many 8-bit cards) will  mis-decode  references  to

ports  that use the upper 6 bits, and may get into

funny states because  they  think  that	they  are

being  addressed	when they are not.  It is recom-

mended that this option be used initially  if  any

8-bit cards are present in the system.

X Version 11Release 6.3 (XFree86 3.2/SuperProbe 2.12)		1

SuperProbe(1)					SuperProbe(1)

-excl list

SuperProbe  will	not  attempt  to  access any I/O

ports on the specified exclusion list.  Some video

cards  use  rather non-standard I/O ports that may

conflict with other cards installed in  your  sys-

tem.   By specifying to SuperProbe a list of ports

already in use, it will know that there cannot  be

any  video  cards  that use those ports, and hence

will not probe them (which could otherwise confuse

your  hardware).	The exclusion list is specified

as a comma-separated list of  I/O  ports	or  port

ranges.	A  range is specified as "low-high", and

is inclusive.  The ports can be specified in deci-

mal,  in	octal	(numbers begin with '0'), or hex-

adecimal (numbers begin with '0x').

-mask10 This option is used in combination with -excl.  It

tells  SuperProbe  that when comparing an I/O port

under test against the exclusion	list,	the  port

address	should	be  masked  to	10 bits.  This is

important with older 8-bit cards that only  do  10

bit  decoding,  and for some cheap 16-bit cards as

well.  This option is simply a  less-drastic  form

of the -no16 option.

-order list

This  option  specifies	which chipsets SuperProbe

should test, and in which order.	The list parame-

ter  is	a  comma-separated list of chipset names.

This list overrides the built-in	default  testing

order.	To find the list of acceptable names, use

the -info option described below.  Note that items

displayed  as  "Standard	video	hardware" are not

usable with the -order option.

-noprobe list

This options specifies which  chipsets  SuperProbe

should not test.	The order of testing will either

be the default order, or that specified	with  the

-order option described above.  The list parameter

is a comma-separated list of  chipset  names.   To

find  the  list of acceptable names, use the -info

option described below.	Note that items displayed

as  "Standard  video hardware" are not usable with

the -noprobe option.

-bios base

This option specifies the  base	address	for  the

graphics-hardware  BIOS.	By  default, SuperProbe

will attempt to locate the BIOS base  on	its  own

(the  normal  address is 0xC0000).  If it fails to

correctly locate the BIOS (an error  message  will

be  printed  if this occurs), the -bios option can

be used to specify the base.

X Version 11Release 6.3 (XFree86 3.2/SuperProbe 2.12)		2

SuperProbe(1)					SuperProbe(1)

-no_bios

Disallow reading of the video BIOS and assume that

an  EGA	or  later (VGA, SVGA) board is present as

the primary video hardware.

-no_dac Skip probing for the RAMDAC type when an (S)VGA is

identified.

-no_mem Skip  probing  for  the	amount of installed video

memory.

-info   SuperProbe will print out a  listing  of	all  the

video hardware that it knows how to identify.

EXAMPLES

To  run	SuperProbe  in its most basic and automated form,

simply enter:

SuperProbe

Note - you may want to redirect stdout to a file when  you

run  SuperProbe	(especially  if	your OS does not support

Virtual Terminals on the console).

However, if you have any 8-bit cards installed, you should

initially run SuperProbe as:

SuperProbe -verbose -no16

(the  -verbose  option  is  included  so	you can see what

SuperProbe is skipping).

Finer granularity can be obtained with an exclusion  list,

for example:

SuperProbe -verbose -excl 0x200,0x220-0x230,0x250

which  will  not	test for any device that use port 0x200,

ports 0x220 through 0x230, inclusive, or port  0x250.   If

you have any 8-bit cards installed, you should add -mask10

to the list of options.

To restrict the search to Western Digital, Tseng, and Cir-

rus chipset, run SuperProbe as follows:

SuperProbe -order WD,Tseng,Cirrus

BUGS

Probably	a  lot at this point.	Please report any bugs or

incorrect identifications to the author.

It is possible that SuperProbe can lock up  your	machine.

Be  sure	to  narrow the search by using the -no16, -excl,

and -mask10  options  provided  to  keep	SuperProbe  from

X Version 11Release 6.3 (XFree86 3.2/SuperProbe 2.12)		3

SuperProbe(1)					SuperProbe(1)

conflicting with other installed hardware.

SEE ALSO

The  vgadoc3.zip documentation package by Finn Thoegersen,

available in the MS-DOS archives of many FTP repositories.

Programmer's  Guide  to	the EGA and VGA Cards, 2nd Ed, by

Richard Ferraro.

AUTHOR

David E. Wexelblat <dwex@xfree86.org>

with help from David  Dawes  <dawes@xfree86.org>	and  the

XFree86 development team.

X Version 11Release 6.3 (XFree86 3.2/SuperProbe 2.12)		4