SHIM Test Program

=================

Usage: test_zkshim [commandfile] (if commandfile is not specified, then will default to "test_zkshim.in"

Commandfile syntax:

the commandfile contains a set of keywords (and corresponding key values) that are used to define values and

perform actions.

the following keywords are defined (if followed by an "equals" sign, then a key value must be specified - no blanks allowed)

HOST=hostname

PORT=port

BINDSOCK

CONNECTSOCK

TYPE_OF_SERVICE=type-of-service-value

PROTOCOL={UDP|TCP}

SOURCE_ADDRESS=nnn.nnn.nnn.nnn

SOURCE_ADDRESS_MASK=nnn.nnn.nnn.nnn

DESTINATION_ADDRESS=nnn.nnn.nnn.nnn

DESTINATION_ADDRESS_MASK=nnn.nnn.nnn.nnn

SOURCE_PORT=portno

DESTINATION_PORT=portno

USER_ID=userid

PROCESS_ID=processid

SESSION_ID=sessionid

CLEARDEF

ADDRULE

DELRULE

SENDSOCK

SENDSHIM

Test #1 - Add/Remove/Clear rules

---------------------------------------

Try defining the following command file (named TEST_ZKSHIM.IN) for example:

HOST=frankf

PORT=3981

SOURCE_ADDRESS=0.0.0.0

SOURCE_ADDRESS_MASK=0.0.0.0

DESTINATION_ADDRESS=10.16.128.115

DESTINATION_ADDRESS_MASK=10.16.128.115

SOURCE_PORT=0

DESTINATION_PORT=3981

USER_ID=56760

PROCESS_ID=0

SESSION_ID=0

DELRULE

ADDRULE

ADDRULE

DESTINATION_ADDRESS=10.16.128.30

DESTINATION_ADDRESS_MASK=10.16.128.30

DELRULE

DESTINATION_ADDRESS=10.16.128.115

DESTINATION_ADDRESS_MASK=10.16.128.115

DELRULE

ADDRULE

DESTINATION_ADDRESS=10.16.128.30

DESTINATION_ADDRESS_MASK=10.16.128.30

ADDRULE

DESTINATION_ADDRESS=127.0.0.1

DESTINATION_ADDRESS_MASK=127.0.0.1

ADDRULE

then,

./test_zkshim TEST_ZKSHIM.IN produces the following:

ioctl ZKIOC_DEL_RULE: No such process

rule added successfully

ioctl ZKIOC_ADD_RULE: File exists

ioctl ZKIOC_DEL_RULE: No such process

rule deleted successfully

rule added successfully

rule added successfully

rule added successfully

also, the file /proc/net/zkshim should contain the following:

Unit            TOS     Proto   SourceIP/Mask           Dest. IP/Mask                   SrcPort DstPort UID     PID     SID

CADB9260        00      11      00000000/00000000       7F000001/7F000001               0       3981    56760   0       0

CADB9260        00      11      00000000/00000000       0A10801E/0A10801E               0       3981    56760   0       0

CADB9260        00      11      00000000/00000000       0A108073/0A108073               0       3981    56760   0       0

Test #2 - Capturing Outbound Packets

-----------------------------------------

2.1 Using a regular socket, send a packet that fits rule A : should be

captured.

Assuming that the test_zkshim process from Test #1 is running,

define a new command file (TEST_ZKSHIM.IN2)

HOST=frankf

PORT=3981

CONNECTSOCK

PROTOCOL=UDP

SOURCE_ADDRESS=10.16.128.30

SOURCE_ADDRESS_MASK=0.0.0.0

DESTINATION_ADDRESS=10.16.128.115

DESTINATION_ADDRESS_MASK=0.0.0.0

SOURCE_PORT=0

DESTINATION_PORT=3981

USER_ID=0

PROCESS_ID=0

SESSION_ID=0

SENDSOCK

DESTINATION_ADDRESS=127.0.0.1

SENDSOCK

then,

./test_zkshim TEST_ZKSHIM.IN2 will send two packets which should be intercepted

the receiving process should display the following:

received 60 bytes from shim

0000  45 00 00 3c f4 19 00 00 40 11 71 91 0a 10 80 73  E..<t...@.q....s

0010  0a 10 80 73 04 62 0f 8d 00 28 eb 77 45 00 00 20  ...s.b...(kwE..

0020  00 00 00 00 40 11 66 1c 0a 10 80 1e 0a 10 80 73  ....@.f........s

0030  04 00 0f 8d 00 0c d7 97 00 00 00 00              ......W.....

received 60 bytes from shim

0000  45 00 00 3c f4 19 00 00 40 11 71 91 0a 10 80 73  E..<t...@.q....s

0010  0a 10 80 73 04 62 0f 8d 00 28 eb 77 45 00 00 20  ...s.b...(kwE..

0020  00 00 00 00 40 11 66 1c 0a 10 80 1e 0a 10 80 73  ....@.f........s

0030  04 00 0f 8d 00 0c d7 97 00 00 00 00              ......W.....

Test #3 - Filtering Rules

-------------------------

Vary the defined rules (e.g. destiniation address, process id, etc.)

and verify that packets are/are not intercepted.

Test #4 - Sending Packets Back Up the Stack

-------------------------------------------

Assuming that the test_zkshim process from Test #1 is running,

define a new command file (TEST_ZKSHIM.IN3)

HOST=frankf

PORT=3981

CONNECTSOCK

PROTOCOL=UDP

SOURCE_ADDRESS=10.16.128.30

SOURCE_ADDRESS_MASK=0.0.0.0

DESTINATION_ADDRESS=10.16.128.115

DESTINATION_ADDRESS_MASK=0.0.0.0

SOURCE_PORT=0

DESTINATION_PORT=3981

USER_ID=0

PROCESS_ID=0

SESSION_ID=0

SENDSHIM

then,

./test_zkshim TEST_ZKSHIM.IN3 will send the shim a packet (containing

specified IP and UDP addressing info), and the shim will put it into

the stack, and it should wind up in the receiving socket:

the receiving process should display the following:

received 4 bytes from 10.16.128.30 on UDP socket

0000  00 00 00 00                                      ....

Test #5 - Multiple User

-----------------------

Repeat test #1 running receiver under your userid, and

sender under root and your userid.

if receiver is run under specific userid, then shim will intercept packets

sent by process with same userid only.

Repeat test #1 running receiver under root, and sender under root and your userid.

if receiver is run under root, then shim will intercept packets sent by process

with any userid.