PRESS [arrow keys] to scroll through this document:

Table of Contents:

1. Kern E1

2. Kern E2

3. Cable Configurations

===================================================

Kern E2: (per manual dated: __-__-__)

1. Mode selection:

a. Angles only.

2. Precision Plumbing using instrument tilt angles:

a. turn Compensator On.

b. plunge telescope @ horizon 100 gons.

c. rotate 200 gons around H circle.

d. observe V circle reading difference from 100 gons.

e. remove 1/2 of d. with leveling screws and 1/2 with

telescope vertical tangent screw.

f. rotate 100 gons around H circle.

g. adjust leveling screws to make V reading 100 gons.

h. repeat c. through g. until on.

i. turn compensator off and observe any error.

3. Instrument Parameters from Inst. Keyboard:

a. set instrument to measure H angles in gons, right, cw.

b. set instrument to measure V angles in gons in direct position;

0 up, +100 horizon, +200 earth.

4. Communication settings required for ET:

a. 1200 baud, 1 Parity Bit (even), 2 stop bits.

{this should be the fixed default}

To Exit: PRESS [Alt],[ENTER],[X]

===================================================

===================================================

Kern E1: (per manual dated: __-__-__)

See Kern E2 above.

To Exit: PRESS [Alt],[ENTER],[X]

===================================================

===================================================

Typical RS232 Cable Configurations (Ref. Only):

TTL

TTL                             to

Inst          Inst Cord        RS232       Std RS232 Cable      Computer

[E2] [Lemo Conn]-----[DB-25] [ 232TTL ] [DB-25]----------[DB-9] [PC]

male     ^--+V     female

E2/E1 PigTail Inst Cord Circuit (not tested as of 08-17-95):

Pin #        Pin #

LEMO CONN    DB-25 CONN (male)

(3)----------(9)

(3)----------(14)

(1)----------(7)

===================================================

TO EXIT:

If using MS-DOS Editor--> PRESS [Alt],[ENTER],[X]

For Other Editors--> Follow specific editors menus.