_______________________________________________________________________

STACKER NOTE                           Stac Electronics Technical Note

Subject: I Stacked my PS/1 before I reconfigured the way it starts.

Now what?

Tech027- 2/20/92

Page 1 of 1

______________________________________________________________________

Note:  If Stacker has not yet been installed, see Tech020: INSTALLING

STACKER ON AN IBM PS/1.

BACKGROUND:

The PS/1 is a ROM based machine.  It normally boots from its

"Built-In-DOS" on the ROM D: drive and  reads config.sys and Autoexec.bat

from there as well.  The PS/1 can be reconfigured, using"CUSTOMIZE",

to read Config.sys and Autoexec.bat from C: so that the Stacker devices

may be loaded.  If the boot process is not "CUSTOMIZed" before loading

Stacker, the data may appear to be "missing" because the STACKER.COM device

has not been loaded.

A 4 Quadrant system menu is normally loaded from ROM upon boot. It consists

of 1.Information 2.Microsoft works 3. Your software and 4. IBM DOS.  You can

shell to a DOS prompt by pressing <shift F9>. It is installed in Config.sys:

"INSTALL=D:\SHELLSTB.COM".

"CUSTOMIZE How System Starts" is a program normally in the C:\DOS directory,

but is copied into the Stacker drive upon "STACKING... the existing drive".

The program also should be on the DOS diskettes that came with the

machine.  In the Customize program you can configure the computer to

Read the system files, Config.sys, and Autoexec.bat  from C: or D:

Note: THE CUSTOMIZE DEFAULTS CAN BE RESTORED BY POWERING DOWN THE

SYSTEM, HOLDING DOWN THE 2 MOUSE BUTTONS, AND POWERING BACK UP.

RECOVERY:

Note:   You MUST have the DOS program diskette containing CUSTOMIZ.EXE

or a bootable floppy to proceed.  Nothing can be done until you have

one of these!

If you have the DOS program diskette:

1. Insert the DOS diskette in A: and Run the CUSTOMIZE program from the

Dos Quadrant.  (You can also <shift> F9 to get to a prompt and run

A:Customiz.)

If you do not have the DOS diskette but have a bootable system

diskette:

1. Insert the system diskette in A: and from the system menu press

Alt+SysRq to force the system to boot from A: (the D: Rom drive

will not be accessible after this.)

2. Copy config.sys to A: and reboot

3. From the DOS directory on the Stacker drive (C:) run CUSTOMIZ.

Running CUSTOMIZE:

1. The third line from the bottom of the screen says "Read

Config.sys:". Click on "from Disk"

2. The next line down says " Read Autoexec.bat". Click on "From Disk"

3. The last line says "Disk to Read from". Click on C:

4. Press <ENTER> to save changes and exit.

5. Reboot the machine.

6. This will bring you to a C: prompt.  Edit the Config.sys on C:\ and

add the line:

Install=D:\SHELLSTB.COM to restore the system Quadrant menu.

7. Restart the system for changes to take affect.