D E C O D I N G   A   V O C - F I L E

-------------------------------------

v1.02

When you open a VOC file, TAPER will pop up a requester where you can adjust

the values used by the decoding routines.

Most of these can generally be left alone.

As TAPER is a full tape handling program, you can see the results directly.

I'll explain what each options does and how it compares to the switches in

VOC2TZX from Tomaz Kac.

In fact, most of this documentation was derived from VOC2TZX.

While reading this document, you will see that I refer to 'T-states'.

A 'T-state' is the counting value as used by the Z80 processor itself and is the

time length of 1 clock pulse. As the Z80 in a ZX Spectrum runs at 3.5 MHz,

1 T-state has a duration of 1/3,500,000 seconds.

First of all, let's see how a tape block is organised:

1. A block starts with a pilot signal. This 'tells' the loading routine that a

new block is starting. The Spectrum ROM needs this pilot tone to be more than

1 second in duration.

2. After the pilot come 2 sync pulses, which are significantly smaller than a

pilot pulse. This marks the end of the pilot.

3. Then comes the datablock. 1 bit of data is composed of 2 equal pulses.

The length of the pulses determines whether it's a '0' or a '1' bit.

Usually, a '1' bit pulse will be twice as long as a '0' bit pulse.

The amplitude of each pulse is not important, as the loading routines are

'edge-triggered' rather than 'level-triggered'.

The datablock itself is composed of the following parts:

- The first byte is the flag byte. The ROM uses this to make the difference

between a 'header' block and a 'data' block.

- The last byte is the parity byte. This is an 8-bit XOR of all previous

bytes, including the flag byte. While loading the data, this parity is

calculated again and matched with this last byte. If they match, loading

was successful, otherwise you get the dreaded 'Tape Loading Error'.

- The rest is the actual data, which is stored in memory.

You may wonder why there are so many variables to decoding. This has several

reasons. First of all, the quality of the analogue sound signal is worse for

older tapes. This is caused by tape wear-off and is exactly why this program

may become important for you! Both the signal strength and the signal/noise

ratio determine how easy it will be to determine what is what on the tape.

Also, the used sample rate has a huge impact. The pulses used to encode the data

on the tape are very short. Therefor, the sampling routine in the Spectrum ROM

uses a sample rate of 60 kHz. This is more than you will need, but will of

course give the best results.

To give you a quick idea, let me tell you this:

The ROM saves a '0' bit as 2 pulses of 855 T each. As 1 T = 1/3,500 ms., this

results in 0.244 ms. (or 0.000244 s.) for a pulse! That's not a whole lot!

And over 70% of the games had some sort of turbo loader, where the pulses for

each bit are even shorter.....

If you use a sample rate of 20 kHz, each sample represents 175 T.

Compared to 855 T, you will see that it makes quite a difference if the saved

data is 1 sample off. And that's for normal speed.

If you use a sample rate of 44.1 kHz, each sample represents around 79 T.

The program TAP2VOC (which comes with the registered version of Z80) uses a

sample rate as low as 4098 Hz. Each sample is now 854 T.

See what happens if you are only as much as 1 sample off!

You will have the best chances if you use a sample rate of 44.1 kHz

(CD quality). This will result in quite a large VOC file, but you will need it

only once. TAPER can then process the VOC file and create tape files of

(usually) some 50 Kb.

There is no need to 'cleanup' a VOC file first, as the decoding routines do a

pretty good job on noise reduction (elimination).

Usually, the best results are found when sampling with READVOC or READSB (which

come with the registered version of Z80), but there is no need to use them.

Any 8-bit mono VOC file will be accepted.

PILOT DETECTION

---------------

Minimum Pulse (T)

Maximum Pulse (T)      : The length of a pulse must be between these values to

be acceptable as a pilot pulse. You may have to

decrease this range when your tape contains much noise,

although this problem is solved better with the

following option.

Min Num Pulses         : This is the minimum number of consequetive pulses that

must have been read to accept the signal as a pilot.

(the /pilot switch in VOC2TZX)

Pulse Threshold (%)    : This determines how much percent two consequetive

pulses may differ to be accepted as continuing wave.

It is used to determine the end of the pilot.

BIT DETECTION

-------------

Minimum pulse (T)      : Before the actual bit-timings have been found, this

value is used to determine the minimum length of one

bit. If a read pulse is smaller than this value, TAPER

assumes there is a small distortion in the bitstream

and adds the next pulse to the bit as well.

When the actual bit-timings have been found, one half-

pulse of a '0' will be used instead.

Threshold '0'/'1' (%)  : Before the second bit-timing has been found, this

value determines the minimal pulse difference to make

a pulse qualify for the other bit-timing.

(the /diff switch in VOC2TZX)

Lossy Detection (?)    : Usually, TAPER will use a middle-pulse value to find

out if a pulse is a '0' or a '1' bit. If you turn this

option on, the absolute bit-timings will be used

instead. The signal tolerance will then be lower.

(turned off is the /middle switch in VOC2TZX)

Use '1' = 2 * '0' (?)  : Usually, TAPER will simply use the timing values as it

detects them. Most custom loaders (and the ROM itself)

have a '1' pulse twice as large as a '0' pulse.

Turn this option on if the tape quality was so bad that

this was not automatically detected.

(turned off is the /noapprox switch in VOC2TZX)

END DETECTION

-------------

Single Pulse ('1')     : If a pulse is read that is more than 'this value' times

a '1' pulse, the end of the block has been reached.

Beware that a lot of tapes have small distortions, so

the next option is usually better.

Joined Pulse ('1')     : As a bit is built from 2 pulses, 2 pulses are read at a

time and their average is used for decoding.

Usually, due to sample rates, both read pulse lengths

are not the same, but the average value is good enough

to determine the correct bit.

A block is always ended in one of two ways:

- a pause

- the next pilot

For 20kHz tapes, 1.2 is usually a better value.

OTHER

-----

Threshold Standard (%) : This determines how close the decoded values (pilot,

'0' and '1' bits) must be to be detected as a normal

speed (ROM values) block.

(the /std switch in VOC2TZX)

Drop Partial End (%)   : Sometimes, the decoded block has some bits following

it (less than 8). These are rarely needed, but may be

read due to a too high tolerance (end detection).

Turn this option on if you're not interested in these

bits.

(the /ignore switch in VOC2TZX)

DECODING SCHEMES

----------------

Unlike in VOC2TZX, the /slskip switch (which detemines how many normal speed

blocks are in front of the special blocks) is never needed. TAPER is intelligent

enough to determine this.

The following schemes are currently supported:

(none)                 : Means that the values must all be determined from the

VOC file. This can be use only for ROM saved programs

or 'normal' turbo speed programs.

Force ROM Values       : Will force the ROM timing values and decode the VOC

with these. This option implicitely turns the

'Drop Partial End' option on.

(the /rom switch in VOC2TZX)

BleepLoad              : You will recognise the BleepLoader by the following

things: it always writes the name of the game on the

screen after the BASIC block is loaded. After the

BASIC, there is another CODE block and then the

BleepLoad follows. This consists of small blocks (with

sizes around 250 bytes) with a very small pilot tone.

There is no interleave between the blocks.

While loading, it writes the block number in hex on

the screen (like 'LOADING 1F'). If there was an error,

you can reload the block with the error.

There are always 2E blocks before the picture and

around 8E blocks after it (this may vary for

different games of course).

(the /bleep switch in VOC2TZX)

Alkatraz               : The Alkatraz protection scheme uses almost standard

tape encoding. This is how you recognise that this

encoding is used:

- The 'Program: ' prompt is not displayed when the

BASIC block loads. Instead only some 5 letters of

the filename are displayed AT 1,1 position;

If you look at the BASIC part after the decoding,

you will see an 'Alkatraz Protection System'

message in it.

- After the BASIC comes a CODE block;

- Then there's a pause for about 12 seconds, while

you hear noise on the tape;

- It then starts loading the screen in some custom

very nice fashion, while the border stays black.

The pilot tone for this block is VERY small (around

240 pulses) and is automatically extended to simulate

the noise from the tape.

- After the screen comes the rest of the block while a

counter with scrolling numbers counts down.

Again with a black border.

It is not necessary that the parity bytes are correct

(as they are not used).

A lot of SpeedLock protected games have the following text line in the last

BASIC block before the turbo blocks: 'Protected by SPEEDLOCK'.

SpeedLock 1            : This one is the first SpeedLock protection, used in

most of the older games (like Fighter Pilot).

The loading itself looks like normal highspeed loading,

but has one distinct give-away: the so-called

'clicking' pilot tone, which means that the pilot is

continuously interrupted with a small distortion.

The border colors are the same as normal tape blocks.

It is not necessary that the parity bytes for the turbo

blocks are correct (as they are not used). The last

byte can be less than 8 bits, so be careful with the

'Drop Partial End' flag.

(the /slock1 switch in VOC2TZX)

SpeedLock 2            : This is the successor of Type 1 and features the same

clicking pilot. The difference can be seen in the

border colors: red/black for the pilot and blue/black

for the data. The second turbo block is composed of

several (small) sub-blocks, which are glued together

with an awkward mid-sync signal.

All parity bytes must be correct for a SpeedLock 2

game to load! The last byte may be less than 8 bits.

(the /slock2 switch in VOC2TZX)

SpeedLock 3            : You will recognise this protection scheme by its very

SHORT BASIC block and a very LONG CODE block (8Kb).

After this, the decryption stage follows for about 25

seconds. During this stage, the border flashes either

with red/black or multicolor (all colors) stripes.

Depending on this, the variation is determined.

During the decryption, you will hear noise from the

tape. Variation 2 NEEDS this noise in order to decrypt

correctly!

After this stage come the two (very encrypted) turbo

blocks:

- First block: loading stripes are red/black for the

pilot tone and blue/black for the data. This block

is very short (some 200-300 bytes);

- Second block: same loading stripes, but the block

length is the entire 48K or even more (for 128K

games). As in SpeedLock 2, this block is composed of

several (small) sub-blocks with an awkward mid-sync

signal.

When the loading picture pops on the screen, the border

goes black and the loading timer starts its countdown

in the format '1m32s10'.

There should be no partial end byte and the parity

should be correct. For the first turbo block this is

simply the last byte, but for the second turbo block

(with the sub-blocks) this is an overall parity byte,

which is the last byte of the last block.

(the /slock3 switch in VOC2TZX)

SpeedLock 4            : This one is VERY similar to SpeedLock 3. The difference

is as follows: There is only one BIG BASIC block (4Kb).

There is almost no pause between the BASIC block and

the first turbo block (the decryption is much faster).

The loading speed is somewhat lower.

Because of this, the mid-sync pulses can sometimes

not be detected. So, there could be a partial end byte

and a wrong parity in the second turbo block while the

game will still successfully load!

(the /slock4 switch in VOC2TZX)

Digital Integration    : This scheme was used by the company of the same name.

It can be seen very easily, as the turbo speed blocks

(after the single small BASIC block) have a pilot

signal that is composed of 2 parts.

The first part is fairly long (some 2.5 seconds), while

the second part is very short and has a lower pitch.

The turbo blocks are heavily encrypted, so don't worry

if the screen looks funny.

(the /diload switch in VOC2TZX)

HINTS

-----

Q: The decoder splits some blocks when there should be only one block. This

usually means you have a lot of blocks and they all have bad parity bytes.

A: The end-of-block detection is set too tight. Increase these values slightly.

Mind though, that when you increase them too much, the following problem can

occur.

Q: The decoder 'glues' blocks together.

A: The end-of-block detection is set too loose. Decrease these values slightly.

This can easily happen when the sample rate is 20kHz or lower.

You can see this after decoding by using the menu option 'View as...' and

choose 'Memory image'. You will probably be able to see the real blocks,

with blocks of 'FF' bytes in between.

Q: The blocks get recognised as custom speed blocks, while they actually are

normal speed blocks.

A: You can increase the 'Threshold Standard' value percentage. Or, if the entire

tape only contains normal speed blocks, you could use the decoding scheme

'Force ROM timing'.

Q: The block gets converted wrong (i.e. loading screen is all garbled).

A: There can be many possible answers to this problem. The quality and sample

rate of the VOC file determines how easy it will be to convert.

You could try to turn 'Lossy Detection' on, increase the 'Threshold '0'/'1''

or turn 'Use '1' = 2 * '0'' on.

Mind though, that the turbo blocks in SpeedLock 2 through 4 are supposed to

be garbled! This is because they are crypted. You could try to set the menu

option 'Auto-decrypt' on. If you get a correct screen now, the block was

decoded correctly. However, some may still look wrong, because they use a

different encryption scheme.

Q: After decoding, I get a bright red data entry.

A: This means there was a normal (ROM) speed datablock, but with an incomplete

last byte. It is printed in bright red, as this partial byte will not be

saved, unless saving as a VOC file again.

Usually, these bits aren't needed anyway, but if you really want to keep

them, you must convert the block to custom speed (View entry info -> Edit)

Q: When I decode a SpeedLock 3 or 4 file, I get dozens of small, bad, blocks

between the normal speed part and the first custom (Group) part.

A: SpeedLock 3 and 4 programs have special noise on the tape. This noise is

heard when the loader decrypts. Parts of this noise are decoded as data.

You'd best increase the 'Minumum Pilot Pulse' length to 500 or up.