(******CONV2SND.DOC******)

*                      *

*  CONV2SND.PAS: 2.00  *

*                      *

*   February 3, 1993   *

*     Kenneth Udut     *

*    Revised 6/22/94   *

*      J.L. Hayes      *

*                      *

(******CONV2SND.DOC******)

CONV2SND.EXE, CONV2SND.PAS, CONV2SND.DOC, CONV2SND.HST HEREBY ARE

RELEASED INTO THE PUBLIC DOMAIN.  UNLESS SPECIAL CIRCUMSTANCES WARRANT

OTHERWISE, EVERY SOFTWARE PRODUCT CREATED BY (_KENNETH UDUT_) WILL:

1. BE RELEASED INTO THE PUBLIC DOMAIN, FREELY COPYABLE AND CHANGEABLE.

2. INCLUDE THE SOURCE CODE.

AS THE AUTHOR OF (_CONV2SND_), I WOULDN'T MIND CREDIT FOR MY WORK.

HOWEVER, IT IS *NOT* A NECESSITY, SO DON'T WORRY ABOUT IT! :)

[Note: My modifications are also public domain.  Ken wrote much of the

following, but I have changed a lot of it to match version 2.00. - J.L.

Hayes]

Now enough of this screaming!

1.  CONV2SND is a program with one thing in mind:  To open the worlds

of digitized sounds created on other machines and by other methods

to anyone using a Tandy 1000 SL, TL, RL, RLX, 2500 RLX, RSX and

every other machine that uses:

SOUND.PDM

Ken wrote: "I wish that Tandy had created something a few years ago

that would have done this, as it might have kept the Tandy's with the

superb 8-bit DAC a little more in step with the SoundBlaster, the

Macintosh, the Amiga and the Atari ST, all of which are able to create

superb digitized sounds and have programs that allow you to share them

with one another.

"So, here it is."

2.  To start the program, type:

CONV2SND <infile>

or:

CONV2SND <infile> <outfile>

The process will then take place automatically.  Filenames can include

drive and path if you want.  If you use the first form, the output

filename will be the same as the input filename, including drive and

path if present, but with an ".snd" extension attached.  If you use the

second form, the output filename will be what you specified.  If the

output filename does not have an extension, it defaults to ".snd".

There is no default for the input file, so you have to specify the

extension for it.  If you want the output file not to have an

extension, end its name with a period.

As of version 2.00, CONV2SND can read and understand .wav headers.  The

.wav will be detected by the header, not by the filename.  CONV2SND

will check that the .wav is an 8-bit mono file and display some advice

if it isn't (see below).  If the .wav is 8-bit mono, CONV2SND will read

the sampling rate from the header and check to see if it is within 5%

of one of the three rates supported by Sound.pdm.  If so, the sampling

rate will be taken from the .wav header, and you won't be asked about

that.  If not, you will be told to resample the file.  There is a

program available to do this, discussed below.  If the .wav header is

invalid, you will be given the option of continuing as if the file were

not a .wav.  I don't know how useful that will be.

You will be asked a couple of questions.

A) Asks about the sampling rate (speed of playback).  You won't be

asked this question if CONV2SND is able to determine the sampling

rate from the .wav header.

These are the options (Ken's notes):

5500 is a slow sampling rate, often used for speech purposes or some

games.  [5500 seems to be rare nowadays. - JLH]

11000 is the the most common sampling rate.  It is #2 on the

selection list and the one you will want to try first.  The

only reason it is so popular is that it is of sufficient

quality for music and such.  5500 really isn't.  WAV files are

usually sampled at 11000 samples per second.

22000 is most often used with the Macintosh Sound files.  It is of a

higher quality, but it does take up more disk space, which is

why it is the 2nd most popular sampling rate to record at.

[This rate is becoming more common. - JLH]

If you screw up and select the wrong one, that's okay!  Just redo the

procedure and select a different rate next time!

[It should be mentioned that 44000, or 44kHz, is an increasingly

common rate.  This is the rate that audio CD's are sampled at.  The

Tandy sound chip is perfectly capable of going that fast, but

Sound.pdm does not support that speed.  I have written a .wav player

for the Tandy chip that will play .wav's at 44kHz - see Snd2wav.doc.

Included here with CONV2SND is a utility called Ppwav that can cut

the sampling rate of these .wav's down to 22kHz so that CONV2SND can

convert them.  See below. - JLH]

B) Asks for the "name" of the sound.

This isn't the file name.  This is the name that appears when you

are in SOUND.PDM, right next to "Name: ", in the right hand box.  If

you don't specify a name, none will be used.  This is a change from

version 1.98, which required you to enter a name.  It is a good idea

to specify a name anyway, in case the name of the file gets changed

at some point in the future.

You can read the name of the sound by typing DEBUG filename.snd and

then "d".  You will see the name of the sound file in the right hand

side of the screen.  (NOTE: "q" gets you out of DEBUG)

C) Asks for your name.  "No real reason yet, but I'm thinking of one!"

writes Ken.  Version 1.98 required you to enter your name; this

version will call you "CONV2SND user" if you don't enter a name.

A new file with your sound will be created.  This is the DeskMate SOUND

version of the other digitized file.

3.  You will need to do the following after the file is created:

* Go into SOUND.PDM, and load the new .SND file (F2, Open)

* Play the file and see if the conversion was done right.  If the file

sounds like Alvin the Chipmunk, the sampling rate you gave was too

low; go back and run CONV2SND again with a higher rate.  If it sounds

like Lurch on "The Addams Family," the rate is too high, and you need

to try a lower one.  (That is, unless the file *is* a recording of

Lurch! :-)  If the file sounds like nothing at all, just static, it

may not be an 8-bit mono PCM file.  You will have to use a program

called Sox to convert it to one before CONV2SND can use it; see

below.

If the input file was a .wav, you should not have this problem.  If

the .wav is of a type that can't be converted (most can), CONV2SND

will tell you why and halt without creating an output file.

* If the sampling rate seems right, carefully remove any noise at the

beginning of the file, using a very small increment (use "sample").

Beginning noise, if present, represents the header of the original

sound file; it is not actual sound.  There are a few types of sound

files that don't have a header ("raw" files made by Sox are one - see

below), but most do.  The noise will be a short click or pop.

Again, if the input file was a .wav, the .wav header will be deleted

automatically, and this problem will not arise.

* Hit F2 and Save the file.

Example: ROCKY.WAV is converted into ROCKY.SND

After listening to the file to check the conversion and getting

rid of the little beginning noise, hit F2, and Save.  Exit

SOUND.PDM if you don't want to do more with the file at that time.

Here are some problems you may have, and what to do about them:

* If the file sounds like nothing at all, just static, it may not be an

8-bit unsigned mono PCM file.  That means the sound data in the file

is not the right type for SOUND.PDM, and it will have to be converted

to 8-bit unsigned mono PCM before CONV2SND can use it.  There is a

program called Sox that can tell you what kind of file it is and do

the conversion.  As of this writing (6/22/1994), Sox is available by

ftp from:

oak.oakland.edu

Get file:

/pub/msdos/sounds/sox7dos.zip

and read the documentation that comes with Sox.  You will need to

tell Sox to make its output file a RIFF WAVE file, size "byte," "1

channel" and select a sampling rate of 5500, 11000, or 22000.  Use

the "verbose" option with the "stat" effect to find out what the

sampling rate of the original file is; if it's close to one of those

three, leave it alone.  CONV2SND will accept sampling rates within 5%

of 5500, 11000, or 22000.  It is best not to resample if you don't

have to since it introduces noise.  Warning: Sox is a little

confusing to use at first.

As an alternative to Sox, this distribution includes a utility called

Ppwav that operates only on .wav files.  Ppwav can convert wav's with

more than one channel to mono and larger sample sizes to 8-bit.  It

can also cut the sampling rate of a file in half.  Sox does all that

Ppwav does and more, but Ppwav is much faster and easier to use.  If

you have a .wav that can't be converted because of one of the

problems that Ppwav can solve, CONV2SND will refuse to convert the

file and tell you to use Ppwav.  See the separate documentation file

on Ppwav.  Note that there are rare types of .wav's that neither Sox

nor Ppwav can convert.

* If you've tried all 3 sampling rates, 5500, 11000, and 22000, and

none of them seems to be right, you will have to resample the sound

to change the sampling rate to one of those three.  SOUND.PDM can

only use one of those 3 rates.  Sox (see above) can be used to do

this.  Changing the sampling rate will introduce noise, unfortun-

ately; change to a sampling rate that is an integer multiple of the

original to minimize the noise.

Other sampling rates that I have seen are 8000, 15000, and 44000.

Sun .au files are usually sampled at 8000 (note that Sun .au is not

PCM but mu-law - Sox can convert these to .wav).  If the input file

is a .wav, CONV2SND will refuse to convert the file if the header

indicates that the sampling rate is not one of the three that

Sound.pdm supports.

* If CONV2SND tells you the disk where the output file goes is full,

you will need to specify another drive for the output file, one with

more space on it.  If you have a single-floppy system, you can

specify drive B:, and DOS will prompt you to swap disks when needed.

It will be awkward to do it this way, but it will work.

* If Sound.pdm tells you the .snd file won't fit in memory, there is

unfortunately not much you can do.  If your system can take

additional DOS memory, you can install additional memory if you

haven't already.  This is a very cheap upgrade and highly

recommended; check your owner's manual to find out how.  I'm

referring to systems with less than 640k of DOS memory; I doubt that

Sound.pdm will use expanded or extended.

That's it!  Ken's phone (908) 241-6246 is open for your use up 'till about

11pm or so.  He is kudut@ritz.mordor.com on the Internet, and Kenneth Udut,

170 East Clay Avenue, Roselle Park, NJ USA 07204-2050.

Note: Ken hasn't worked with this program in a while, but I expect he will

continue to act as a clearinghouse for the various modifiers, such as

myself.  I am responsible for the modifications from version 1.98 to

version 2.00, including much of the above text.  My phone is (207) 866-

7903.  I'm tvdog@delphi.com on the Internet.  Snailmail: Jeffrey L. Hayes,

130 Forest Ave., Lot 1, Orono, Maine 04473.

"Thanks for reading and hope you NOT ONLY enjoy using it, but also play

around with the source code and work with it!

"I want to see a world where source code is freely distributable, where

there isn't all that "data-hiding" and all that.  I would hate to see a

society where the 'elite programmers' only knew the secrets of 'real'

programming, while the peons were kept down low...

"I prefer openness and honesty, personally :)

"Talk to you soon!

"--Ken

"February 3, 1993"

FIDONET: Kenneth Udut @ 1:107/900

INTERNET: kudut@ritz.mordor.com

America Online: K Udut

PC-Link: K Udut

CompuServe: INTERNET> kudut@ritz.mordor.com

BITNET: kudut%ritz.mordor.com@yalevm.bitnet

Home Address:

Kenneth Udut

170 East Clay Avenue

Roselle Park, New Jersey 07204-2050

Phone Numbers:

(908) 241-6246

(908) 241-8528  --  FAX number

Can be found on:

The Juice Bar BBS, located in Roselle Park, New Jersey, USA

(908) 298-0764.  I'm user #11, and a VERY VERY active poster!

I also run a list on BITNET.

Y-RIGHTS@SJUVM.BITNET - discussion on the rights of kids and teens.

"Leave me a message somewhere or give me a phone call!  I'll set a pot of

tea out for you and we'll chat :)" - Ken

Also check out the Tandy sound package, now on Simtel!  Play mono or stereo

8- or 16-bit .wav's of arbitrary size at up to 65535 Hz, record .wav's of

arbitrary length at up to 62kHz, convert raw 8-bit sound files to .wav,

convert 16-bit or stereo .wav's to mono 8-bit, convert .snd instrument

files to Amiga .mod samples (including looped samples!), use your Tandy DAC

with programs that support a generic DAC, and play your Tandy like a piano

with the 3-voice chip (chords too!).  All source code included, and the

best, most complete programming information on the Tandy sound that you

will find anywhere!  All free!  How do I get this marvellous package, you

ask?  Just ftp it from:

oak.oakland.edu:/pub/msdos/sound/tspak.zip

- JLH

-----------[technical stuff]------------

Many (but not all) types of sound files are basically identical to DeskMate

.snd.  That is, the actual sound data is encoded in exactly the same way.

The difference lies in the "header."

The header of a file is a small series of bytes, usually 44 in the case of

DeskMate .snd files.  The purpose of a header?

A header tells the program how big the file is, as most programs that read

digitized sound files need to be specially told how many bits of sound they

need to load up.  It also might tell you the "name" or description of the

sound file.  The sampling rate (how fast the sound was recorded and should

be played back) is also usually included.

Christopher Taveres was the first to try to decode the .snd header.

Chris's effort appeared as part of his SOUNDOFF.C program in _PCM_, the

Tandy magazine.  Here's what he came up with:

======================================================================

/* Sound Off!

/* Written by Christopher Taveres

/* Copyright (c) January 1992

/* Falsoft, Inc.

/* PCM

/** Definintion for the header block for Deskmate .snd files **/

struct dmheader {            /* Structure of the header block      */

int marker;           /* Marker bytes - should be 00 1a     */

char note_count;      /* Number of notes in instrument file */

char inst_num;        /* Instrument number                  */

char inst_name[10];   /* Instrument name                    */

int sample_rate;      /* Sampling rate                      */

char filler[16];      /* I don't know what this does        */

unsigned long sample_size;  /* Number of samples in file    */

char filler2[8];      /* More unknown space                 */

======================================================================

Ken noticed that the marker bytes could be 1A 01 instead of 1A 00.  The

number of samples confused him, however.  After fiddling around with

Sound.pdm for a while, I filled in most of the blanks as follows:

TANDY DESKMATE .SND FORMAT NOTES

There are two types of DeskMate .snd files, sound files and instrument files.

Both contain 8-bit unsigned PCM samples.  "PCM" stands for "pulse code

modulation" and just means that the sound wave is measured at fixed intervals

and its level recorded.  It's just like you used to do in high school algebra

when you were asked to graph a function by evaluating it at various points.

With PCM, the samples are integers in a certain range; the Tandy sound chip

uses 0-255.

Sound files are simpler.  These are garden-variety sample files with a fixed-

length header giving the name of the sound, the recording frequency, and the

length of the sound.  Sound files may be recorded at 5500Hz, 11kHz or 22kHz.

Instrument files contain samples as well as frequency and looping information

used by Music.pdm to represent an instrument.  Instrument files provide for

attack, sustain, and decay with several samples having different implied

frequencies and being used by Music.pdm to represent the instrument in

different pitch ranges.  Up to 16 different notes (with 16 different samples)

can be contained in one instrument file.  Instrument files are always recorded

at 11kHz.

Both sound files and instrument files may be compressed in one of two ways,

"music" compression or "speech" compression, or they may be uncompressed.  I

don't know the compression algorithms, but simple file comparison reveals that

"music" and "speech" compression are almost identical.

The DeskMate .snd file header consists of 16 bytes of fixed header information

followed by one or more 28-byte note records.  The sample information, which

may be compressed, follows the header.

The information below is the old-format .snd header used by the 1000-series.

This is the format written by CONV2SND.  The 2500's use a somewhat different

header when writing (see Snd2wav.doc), but they can read the old format.

DeskMate .snd File Format - Fixed Header

----------------------------------------

offset    size      what

------    ----      ----

0         byte      1Ah (.snd ID byte).  Note that 1Ah = <cntrl>-Z, so if

a user attempts to type the .snd file to the screen,

nothing will be displayed.

1         byte      Compression code:  0 = no compression; 1 = music

compression; 2 = speech compression.

2         byte      Number of notes in the instrument file.  1 if sound

file.  There can be up to 16 notes in an instrument

file.  Note:  when an instrument file is created, it

automatically has one unset note placed in it.

3         byte      Instrument number.  0 if sound file; 0FFh if instrument

file with no number set.  Valid instrument numbers in

an instrument file are 1 to 32.  Use this field to

distinguish a sound file from an instrument file.

4         10 bytes  Sound or instrument name.  Name is a null-terminated

string, so there are up to 9 characters of the name,

plus the null byte.

0Eh       word      Sampling rate in samples per second.  Note that although

a sampling rate other than 5500, 11000 and 22000 can be

entered here, Sound.pdm will not actually play at other

rates.

10h       variable  Note records begin, 28 bytes each.  Number of records

given in byte 2 above.

DeskMate .snd File Format - Note Record

---------------------------------------

0         byte      Pitch of the note:  1 = A1 in American Standard Pitch;

2 = A#1; etc.  A1 is lowest note allowed; highest note

allowed is B6 (3Fh).  Sound files have 0FFh here; so do

instrument files with no note set.

Note that Sound.pdm does not designate notes in the

standard manner to the user.  Although A1 and B6 in

Sound.pdm are the same as A1 and B6 in standard pitch,

Sound.pdm starts octaves at A rather than at C (as is

standard).  Thus, middle C, C4 in standard pitch, is C3

in Sound.pdm.

1         byte      Sound files, and instrument files with no pitch set,

have 0 here.  If the pitch is set, this byte is 0FFh.

2         2 bytes   Range of the note, first byte is lower limit, second

is higher limit.  Byte encoding as for offset 0 (i.e.,

01h to 3Fh).  Sound files have FF FF here; so do

instrument files with no range set.

4         dword     Offset in the file where samples for this note begin

(zero-relative), after compression if that was done.

8         dword     If compressed, the length of the compressed data in the

file for this note.  Uncompressed files have 0 here.

0Ch       4 bytes   Unknown.  Set to zero.

10h       dword     Number of samples in the note, after decompression if

necessary.  0 can appear here, meaning the note is not

set, or the sound file is empty.

14h       dword     Number of sample at start of sustain region for the

note, relative to the first (zeroth) sample of the note.

For sound files, or if sustain is not set, this field is

0.

18h       dword     Number of sample at end of sustain region for the note,

relative to the first (zeroth) sample of the note.  For

sound files, or if sustain is not set, this field is 0.

//// end of conv2snd.doc ////