REDIRECT.SYS

(shareware)

A SERIAL PORT DEVICE DRIVER FOR REDIRECTING I/O DURING REMOTE COMMUNICATIONS

Copyright 1990 Infinity Design Concepts Inc

1052 Parkway Drive

Louisville, Kentucky 40217-2333

Voice: (502) 636-1234  Modem: (502) 635-5471

Written by Gary Conway

WHAT IS REDIRECT FOR?

-------------------

REDIRECT is a device driver, loaded via the config.sys file. It is

intended to be used with IDCCOM (tm), but may be used with any

communications package, running under DOS 2.x or above with full

features. With DOS versions 2.x and below, the only COM port that

may be used is COM1 and this MAY NOT be specified on the command

line, since the device driver loading routines do not support

command lines in the CONFIG.SYS file in DOS versions below 3.x.

NOTE: Even though the filename for the device driver is

REDIRECT.SYS, the logical device installed by the

driver is named IDCDOS. It would be much nicer to have

the driver with the same name as the logical device, but

this would confuse DOS when the device driver is

loaded and then trying to copy a fresh copy of it to your

hard drive. The system will lock up since DOS becomes

extremely confused trying to figure out how to copy

to/from the IDCDOS device. This would be the same as

creating a file named AUX, PRN, CON etc. since these

are reserved names for logical devices.

REDIRECT will display all characters that come in the selected

COM port on the local (host) screen and echo all characters sent

to the local console back to the COM port, except those that came

in the COM port as shown in the diagram below.

+----------+              +-------------+

|   HOST   | <------------|  COM port   |

|  SYSTEM  |              +-------------+

|  SCREEN  |                    ^

+----------+                    |

|

+---------------------+         |

| HOST SYTEM KEYBOARD |---------+

| OR PROGRAM  OR DOS  |

+---------------------+

REDIRECT loads into memory at boot time and remains inactive until

activated by a CTTY IDCDOS command. Usually, this command will

appear in a BATCH file, when running a door program or an external

program from within the communications program. REDIRECT, will allow

the  SYSOP to monitor the activity of the caller when in the external

program.

REDIRECT is deactivated by the CTTY CON command, again, usually in a

batch file. An example file would be..

ECHO OFF

CLS

CTTY IDCDOS

ZIPVIEW

CTTY CON

This would allow the communications package to execute the ZIPVIEW (tm)

program, also allowing the SYSOP to visually monitor all that

transpires.

ALTERNATE METHOD

----------------

REDIRECT may also be used with the standard DOS redirection symbols

"<" and ">" to redirect input and output for a device or program.

All output from REDIRECT (chars from local keyboard or from serial

port) are sent to COMMAND.COM as input. All output from COMMAND.COM

is sent to REDIRECT as input, where they are in turn sent to the serial

port. Note that if using COMMAND.COM in a shell to DOS operation

from a host system, the remote system should turn on HALF DUPLEX

so that remote characters are echoed locally on the remote end.

ECHO OFF

CLS

CTTY IDCDOS

COMMAND <IDCDOS > IDCDOS

CTTY CON

This method causes all input and output to and from COMMAND.COM

to be run through the REDIRECT driver.

NOTE: the above method will not work with all programs, so some

experimentation is in order, however, one of the two methods

above will definitely work.

LOADING REDIRECT

--------------

REDIRECT is loaded via the CONFIG.SYS file at boot time. The default

COM port for REDIRECT is COM1, but either COM1 or COM2 may be

specified on the command line, e.g.

DEVICE=REDIRECT.SYS 2

The above line will load REDIRECT using COM2. Note that COM1 and COM2

are hard coded to use the following,

COM 1 = 3F8h base port address (std. COM1 location)

COM 2 = 2F8h base port address (std. COM2 location)

Note that for COM3 and COM4 use, there is no problem as long as

the com base ports used are the same as those listed above.

REDIRECT will work with any program that uses standard DOS input

and output functions.

TWIT KEY

--------

The SYSOP of the host system may use the F1 key to immediately

drop the DTR signal (hang up the modem) anytime after the

CTTY IDCDOS command has been executed and up until the time

the CTTY CON command is executed. The result is that if a caller

is shelled to DOS or running a door type program, the SYSOP may

use the F1 key to terminate the call instantly, if the need arises.

ANSI SUPPORT

------------

The following ANSI sequences are supported directly by REDIRECT.

CUU -  cursor up              - ESC [ Pn A

CUD -  cursor down            - ESC [ Pn B

CUF -  cursor forward         - ESC [ Pn C

CUB -  cursor backward        - ESC [ Pn D

CUP - cursor pos              - ESC [ Pl;Pc H

ED  - erase display           - ESC [ 2 J

EL  - erase line              - ESC [ K

SGR - set graphics rendition  - ESC [ Ps ; ... ; Ps m

SCP - save cursor position    - ESC [ s

RCP - restore cursor position - ESC [ u

REDIRECT is a shareware program. If you find yourself using REDIRECT

on a BBS system or for any other reason on a regular basis, you

are expected to register the product. REDIRECT may not be used

in any connections with commercial ventures or in for-profit

situations without registering your copy of the program.

REGISTRATION FEE: $23.50

Site license users please call for a quotation.

DISCLAIMER

----------

This software and documentation are distributed on an as-is

basis and no warranty of any kind is made. IDC assumes no

responsibility as to the suitability for any purpose and

will assume no responsiblity for any damages of any kind.

The user assumes full responsiblity for any loss or damage

arising from the use or misuse of the software and/or

documentation.

<end file>