Beholder -- The Next Generation

RMON compliant Ethernet monitor

DNPAP group of the Delft University of Technology.

Apr. 13 1994

Beholder TNG MIB Documentation (under construction)

------------------------------

--  to keep SunNet Manager's mib2schema happy

--  Remote Network Monitoring MIB

--  DNPAP MIB Extensions

-- textual conventions

-- This data type is used to model an administratively

-- assigned name of the owner of a resource. This

-- information is taken from the NVT ASCII character set.

-- It is suggested that this name contain one or more

-- of the following:

-- IP address, management station name, network manager's

-- name, location, or phone number.

-- In some cases the agent itself will be the owner of

-- an entry.  In these cases, this string shall be set

-- to a string starting with 'monitor'.

--

-- SNMP access control is articulated entirely in terms of

-- the contents of MIB views; access to a particular SNMP

-- object instance depends only upon its presence or

-- absence in a particular MIB view and never upon its

-- value or the value of related object instances. Thus,

-- objects of this type afford resolution of resource

-- contention only among cooperating managers; they

-- realize no access control function with respect

-- to uncooperative parties.

--

-- By convention, objects with this syntax are declared

-- as having

--

--      SIZE (0..127)

--  to keep SunNet Manager's mib2schema happy

--  the enumerations have been commented out

--  { valid(1),

--    createRequest(2),

--    underCreation(3),

--    invalid(4)

--  }

-- The status of a table entry.

--

-- Setting this object to the value invalid(4) has the

-- effect of invalidating the corresponding entry.

-- That is, it effectively disassociates the mapping

-- identified with said entry.

-- It is an implementation-specific matter as to whether

-- the agent removes an invalidated entry from the table.

-- Accordingly, management stations must be prepared to

-- receive tabular information from agents that corresponds

-- to entries currently not in use.  Proper

-- interpretation of such entries requires examination

-- of the relevant EntryStatus object.

--

-- An existing instance of this object cannot be set to

-- createRequest(2).  This object may only be set to

-- createRequest(2) when this instance is created.  When

-- this object is created, the agent may wish to create

-- supplemental object instances to complete a conceptual

-- row in this table.  Immediately after completing the

-- create operation, the agent must set this object to

-- underCreation(3).

--

-- Entries shall exist in the underCreation(3) state until

-- the management station is finished configuring the

-- entry and sets this object to valid(1) or aborts,

-- setting this object to invalid(4).  If the agent

-- determines that an entry has been in the

-- underCreation(3) state for an abnormally long time,

-- it may decide that the management station has

-- crashed.  If the agent makes this decision,

-- it may set this object to invalid(4) to reclaim the

-- entry.  A prudent agent will understand that the

-- management station may need to wait for human input

-- and will allow for that possibility in its

-- determination of this abnormally long period.

-- A data type to specify times in microseconds

-- The Statistics Group

--

-- Implementation of the Statistics group is optional.

--

-- The statistics group contains statistics measured by the

-- probe for each monitored interface on this device.  These

-- statistics take the form of free running counters that

-- start from zero when a valid entry is created.

--

-- This group currently has statistics defined only for

-- Ethernet interfaces.  Each etherStatsEntry contains

-- statistics for one Ethernet interface.  The probe must

-- create one etherStats entry for each monitored Ethernet

-- interface on the device.

etherStatsTable OBJECT-TYPE

-- The History Group

-- Implementation of the History group is optional.

--

-- The history group records periodic statistical samples from

-- a network and stores them for later retrieval.  The

-- historyControl table stores configuration entries that each

-- define an interface, polling period, and other parameters.

-- Once samples are taken, their data is stored in an entry

-- in a media-specific table.  Each such entry defines one

-- sample, and is associated with the historyControlEntry that

-- caused the sample to be taken.  Currently the only media-

-- specific table defined is the etherHistoryTable, for

-- Ethernet networks.

--

-- If the probe keeps track of the time of day, it should

-- start the first sample of the history at a time such that

-- when the next hour of the day begins, a sample is

-- started at that instant.  This tends to make more

-- user-friendly reports, and enables comparison of reports

-- from different probes that have relatively accurate time

-- of day.

--

-- The monitor is encouraged to add two history control entries

-- per monitored interface upon initialization that describe

-- a short term and a long term polling period.  Suggested

-- parameters are 30 seconds for the short term polling

-- period and 30 minutes for the long term period.

historyControlTable OBJECT-TYPE

-- Ether History table

etherHistoryTable OBJECT-TYPE

-- The Alarm Group

-- Implementation of the Alarm group is optional.

--

-- The Alarm Group requires the implementation of the Event

-- group.

--

-- The Alarm group periodically takes statistical samples from

-- variables in the probe and compares them to thresholds

-- that have been configured.  The alarm table stores

-- configuration entries that each define a variable,

-- polling period, and threshold parameters.  If a sample is

-- found to cross the threshold values, an event is

-- generated.  Only variables that resolve to an ASN.1

-- primitive type of INTEGER (INTEGER, Counter,

-- Gauge, or TimeTicks) may be monitored in this way.

--

-- This function has a hysteresis mechanism to limit the

-- generation of events.  This mechanism generates one event

-- as a threshold is crossed in the appropriate direction.

-- No more events are generated for that threshold until the

-- opposite threshold is crossed.

--

-- In the case of a sampling a deltaValue, a probe may

-- implement this mechanism with more precision if it takes

-- a delta sample twice per period, each time comparing the

-- sum of the latest two samples to the threshold.  This

-- allows the detection of threshold crossings

-- that span the sampling boundary.  Note that this does not

-- require any special configuration of the threshold value.

-- It is suggested that probes implement this more precise

-- algorithm.

alarmTable OBJECT-TYPE

-- The Host Group

-- Implementation of the Host group is optional.

--

-- The host group discovers new hosts on the network by

-- keeping a list of source and destination MAC Addresses seen

-- in good packets.  For each of these addresses, the host

-- group keeps a set of statistics.  The hostControlTable

-- controls which interfaces this function is performed on,

-- and contains some information about the process.  On

-- behalf of each hostControlEntry, data is collected on an

-- interface and placed both the hostTable and the

-- hostTimeTable.  If the monitoring device finds itself

-- short of resources, it may delete entries as needed.  It

-- is suggested that the device delete the least recently

-- used entries first.

-- The hostTable contains entries for each address

-- discovered on a particular interface.  Each entry

-- contains statistical data about that host.  This table

-- is indexed by the MAC address of the host, through

-- which a random access may be achieved.

-- The hostTimeTable contains data in the same format as the

-- hostTable, and must contain the same set of hosts, but is

-- indexed using hostTimeCreationOrder rather than hostAddress.

-- The hostTimeCreationOrder is an integer which reflects

-- the relative order in which a particular entry was

-- discovered and thus inserted into the table.  As this

-- order, and thus index, is among those entries currently

-- in the table, the index for a particular entry may change

-- if an (earlier) entry is deleted.  Thus the association

-- between hostTimeCreationOrder and hostTimeEntry may be

-- broken at any time.

-- The hostTimeTable has two important uses.  The first is the

-- fast download of this potentially large table.  Because the

-- index of this table runs from 1 to the size of the table,

-- inclusive, its values are predictable.  This allows very

-- efficient packing of variables into SNMP PDU's and allows

-- a table transfer to have multiple packets outstanding.

-- These benefits increase transfer rates tremendously.

-- The second use of the hostTimeTable is the efficient

-- discovery by the management station of new entries added

-- to the table.  After the management station has

-- downloaded the entire table, it knows that new entries

-- will be added immediately after the end of the current

-- table.  It can thus detect new entries there

-- and retrieve them easily.

-- Because the association between hostTimeCreationOrder and

-- hostTimeEntry may be broken at any time, the management

-- station must monitor the related hostControlLastDeleteTime

-- object.  When the management station thus detects a deletion,

-- it must assume that any such associations have been broken,

-- and invalidate any it has stored locally.  This includes

-- restarting any download of the hostTimeTable that may have

-- been in progress, as well as rediscovering the end of the

-- hostTimeTable so that it may detect new entries.  If the

-- management station does not detect the broken association,

-- it may continue to refer to a particular host by its

-- creationOrder while unwittingly retrieving the data

-- associated with another host entirely.  If this happens

-- while downloading the host table, the management station

-- may fail to download all of the entries in the table.

hostControlTable OBJECT-TYPE

hostTable OBJECT-TYPE

-- host Time Table

hostTimeTable OBJECT-TYPE

-- The Host Top "N" Group

-- Implementation of the Host Top N group is optional.

--

-- The Host Top N group requires the implementation of the

-- host group.

--

-- The Host Top N group is used to prepare reports that

-- describe the hosts that top a list ordered by one of

-- their statistics.  The available statistics are samples

-- of one of their base statistics, over an interval

-- specified by the management station.  Thus, these

-- statistics are rate based.  The management

-- station also selects how many such hosts are reported.

-- The hostTopNControlTable is used to initiate the generation

-- of such a report.  The management station may select the

-- parameters of such a report, such as which interface,

-- which statistic, how many hosts, and the start and stop

-- times of the sampling.  When the report is prepared,

-- entries are created in the hostTopNTable associated with

-- the relevant hostTopNControlEntry.  These entries are

-- static for each report after it has been prepared.

hostTopNControlTable OBJECT-TYPE

hostTopNTable OBJECT-TYPE

-- The Matrix Group

-- Implementation of the Matrix group is optional.

--

-- The Matrix group consists of the matrixControlTable,

-- matrixSDTable and the matrixDSTable.  These tables

-- store statistics for a particular conversation between

-- two addresses.  As the device detects a new conversation,

-- including those to a non-unicast address, it creates a

-- new entry in both of the matrix tables.

-- It must only create new entries based on information

-- received in good packets.  If the monitoring device finds

-- itself short of resources, it may delete entries as needed.

-- It is suggested that the device delete the least recently

-- used entries first.

matrixControlTable OBJECT-TYPE

matrixSDTable OBJECT-TYPE

-- Traffic matrix tables from destination to source

matrixDSTable OBJECT-TYPE

-- The Filter Group

-- Implementation of the Filter group is optional.

--

-- The Filter group allows packets to be captured with an

-- arbitrary filter expression.  A logical data and

-- event stream or "channel" is formed by the packets

-- that match the filter expression.

--

-- This filter mechanism allows the creation of an arbitrary

-- logical expression with which to filter packets.  Each

-- filter associated with a channel is OR'ed with the others.

-- Within a filter, any bits checked in the data and status are

-- AND'ed with respect to other bits in the same filter.  The

-- NotMask also allows for checking for inequality.  Finally,

-- the channelAcceptType object allows for inversion of the

-- whole equation.

--

-- The channel can be turned on or off, and can also

-- generate events when packets pass through it.

filterTable OBJECT-TYPE

channelTable OBJECT-TYPE

-- The Packet Capture Group

-- Implementation of the Packet Capture group is optional.

--

-- The Packet Capture Group requires implementation of the

-- Filter Group.

--

-- The Packet Capture group allows packets to be captured

-- upon a filter match.  The bufferControlTable controls

-- the captured packets output from a channel that is

-- associated with it.  The captured packets are placed

-- in entries in the captureBufferTable.  These entries are

-- associated with the bufferControlEntry on whose behalf they

-- were stored.

bufferControlTable OBJECT-TYPE

wrapWhenFull(2)    -- FIFO

captureBufferTable OBJECT-TYPE

-- The Event Group

-- Implementation of the Event group is optional.

--

-- The Event group controls the generation and notification

-- of events from this device.  Each entry in the eventTable

-- describes the parameters of the event that can be triggered.

-- Each event entry is fired by an associated condition located

-- elsewhere in the MIB.  An event entry may also be associated

-- with a function elsewhere in the MIB that will be executed

-- when the event is generated.  For example, a channel may

-- be turned on or off by the firing of an event.

--

-- Each eventEntry may optionally specify that a log entry

-- be created on its behalf whenever the event occurs.

-- Each entry may also specify that notification should

-- occur by way of SNMP trap messages.  In this case, the

-- community for the trap message is given in the associated

-- eventCommunity object.  The enterprise and specific trap

-- fields of the trap are determined by the condition that

-- triggered the event.  Three traps are defined in a companion

-- document: risingAlarm, fallingAlarm, and packetMatch.

-- If the eventTable is triggered by a condition specified

-- elsewhere, the enterprise and specific trap fields

-- must be specified for traps generated for that condition.

eventTable OBJECT-TYPE

snmp-trap(3),    -- send an SNMP trap

logTable OBJECT-TYPE

-- Beholder TNG Info Table

-- added by Richard Kooijman, 2 Sept 1992  (RK)

beholderInfoTable OBJECT-TYPE

-- Beholder performance statistics

-- Beholder memory allocation statistics

-- Beholder registration variables

-- Sends a trap including a message to a specific community.

-- Beholder TNG Neuro Table

-- added by Richard Kooijman, 2 Sept 1992  (RK)

-- The Neuro table implements neural networks that

-- can have arbitrary MIB variables as in- and output as

-- long as their type is a number (COUNTER, INTEGER, GAUGE).

-- The neural network can then be trained to derive

-- similar output values if similar input values are

-- applied.

neuroTable OBJECT-TYPE

neuroLayerTable OBJECT-TYPE

neuroLayerNeuronTable OBJECT-TYPE

neuroLayerNeuronValueTable OBJECT-TYPE

-- The Statistics Group

--

-- Implementation of the Dnpap Statistics group is optional.

--

-- The Dnpap statistics group contains statistics measured by the

-- probe for each monitored interface on this device.  These

-- statistics take the form of free running counters that

-- start from zero when a valid entry is created.

--

-- This group currently has statistics defined only for

-- Ethernet interfaces.  Each dEtherStatsEntry contains

-- statistics for one Ethernet interface.  The probe must

-- create one dEtherStats entry for each monitored Ethernet

-- interface on the device.

dEtherStatsTable OBJECT-TYPE

dEtherStatsIFTTable OBJECT-TYPE

-- The DHost Group

-- Implementation of the DHost group is optional.

--

-- The dHost group discovers new hosts on the network by

-- keeping a list of source and destination MAC Addresses seen

-- in good packets.  For each of these addresses, the host

-- group keeps a set of statistics.  The dHostControlTable

-- controls which interfaces this function is performed on,

-- and contains some information about the process.  On

-- behalf of each dHostControlEntry, data is collected on an

-- interface and placed both the dHostTable and the

-- dHostTimeTable.  If the monitoring device finds itself

-- short of resources, it may delete entries as needed.  It

-- is suggested that the device delete the least recently

-- used entries first.

-- The dHostTable contains entries for each address

-- discovered on a particular interface.  Each entry

-- contains statistical data about that host.  This table

-- is indexed by the MAC address of the host, through

-- which a random access may be achieved.

-- The dHostTimeTable contains data in the same format as the

-- dHostTable, and must contain the same set of hosts, but is

-- indexed using dHostTimeCreationOrder rather than dHostAddress.

-- The dHostTimeCreationOrder is an integer which reflects

-- the relative order in which a particular entry was

-- discovered and thus inserted into the table.  As this

-- order, and thus index, is among those entries currently

-- in the table, the index for a particular entry may change

-- if an (earlier) entry is deleted.  Thus the association

-- between dHostTimeCreationOrder and dHostTimeEntry may be

-- broken at any time.

-- The dHostTimeTable has two important uses.  The first is the

-- fast download of this potentially large table.  Because the

-- index of this table runs from 1 to the size of the table,

-- inclusive, its values are predictable.  This allows very

-- efficient packing of variables into SNMP PDU's and allows

-- a table transfer to have multiple packets outstanding.

-- These benefits increase transfer rates tremendously.

-- The second use of the dHostTimeTable is the efficient

-- discovery by the management station of new entries added

-- to the table.  After the management station has

-- downloaded the entire table, it knows that new entries

-- will be added immediately after the end of the current

-- table.  It can thus detect new entries there

-- and retrieve them easily.

-- Because the association between dHostTimeCreationOrder and

-- dHostTimeEntry may be broken at any time, the management

-- station must monitor the related dHostControlLastDeleteTime

-- object.  When the management station thus detects a deletion,

-- it must assume that any such associations have been broken,

-- and invalidate any it has stored locally.  This includes

-- restarting any download of the dHostTimeTable that may have

-- been in progress, as well as rediscovering the end of the

-- dHostTimeTable so that it may detect new entries.  If the

-- management station does not detect the broken association,

-- it may continue to refer to a particular host by its

-- creationOrder while unwittingly retrieving the data

-- associated with another host entirely.  If this happens

-- while downloading the host table, the management station

-- may fail to download all of the entries in the table.

dHostControlTable OBJECT-TYPE

dHostTable OBJECT-TYPE

-- dHost Time Table

dHostTimeTable OBJECT-TYPE

dHostIFTTable OBJECT-TYPE

dHostIFTTimeTable OBJECT-TYPE

-- The DHost Top "N" Group

-- Implementation of the DHost Top N group is optional.

--

-- The DHost Top N group requires the implementation of the

-- host group.

--

-- The DHost Top N group is used to prepare reports that

-- describe the hosts that top a list ordered by one of

-- their statistics.  The available statistics are samples

-- of one of their base statistics, over an interval

-- specified by the management station.  Thus, these

-- statistics are rate based.  The management

-- station also selects how many such hosts are reported.

-- The dHostTopNControlTable is used to initiate the generation

-- of such a report.  The management station may select the

-- parameters of such a report, such as which interface,

-- which statistic, how many hosts, and the start and stop

-- times of the sampling.  When the report is prepared,

-- entries are created in the dHostTopNTable associated with

-- the relevant dHostTopNControlEntry.  These entries are

-- static for each report after it has been prepared.

dHostTopNControlTable OBJECT-TYPE

dHostTopNTable OBJECT-TYPE

-- The Dnpap Matrix Group

-- Implementation of the Dnpap Matrix group is optional.

--

-- The Dnpap Matrix group consists of the dMatrixControlTable,

-- dMatrixSDTable and the dMatrixDSTable.  These tables

-- store statistics for a particular conversation between

-- two addresses.  As the device detects a new conversation,

-- including those to a non-unicast address, it creates a

-- new entry in both of the matrix tables.

-- It must only create new entries based on information

-- received in good packets.  If the monitoring device finds

-- itself short of resources, it may delete entries as needed.

-- It is suggested that the device delete the least recently

-- used entries first.

dMatrixControlTable OBJECT-TYPE

dMatrixSDTable OBJECT-TYPE

-- Traffic matrix tables from destination to source

dMatrixDSTable OBJECT-TYPE

-- Traffic matrix tables with topN data from source to

-- a particular ranking

dMatrixSRankTopNTable OBJECT-TYPE

-- Traffic matrix tables with topN data from destination to

-- a particular ranking

dMatrixDRankTopNTable OBJECT-TYPE

-- The DFilter Group

-- Implementation of the DFilter group is optional.

--

-- The DFilter group allows packets to be captured with an

-- arbitrary filter expression.  A logical data and

-- event stream or "channel" is formed by the packets

-- that match the filter expression.

--

-- This filter mechanism allows the creation of an arbitrary

-- logical expression with which to filter packets.  Each

-- filter associated with a channel is OR'ed with the others.

-- Within a filter, any bits checked in the data and status are

-- AND'ed with respect to other bits in the same filter.  The

-- NotMask also allows for checking for inequality.  Finally,

-- the channelAcceptType object allows for inversion of the

-- whole equation.

--

-- The channel can be turned on or off, and can also

-- generate events when packets pass through it.

dFilterTable OBJECT-TYPE

dChannelTable OBJECT-TYPE

-- The Packet Capture Group

-- Implementation of the Packet Capture group is optional.

--

-- The Packet Capture Group requires implementation of the

-- Filter Group.

--

-- The Packet Capture group allows packets to be captured

-- upon a filter match.  The dBufferControlTable controls

-- the captured packets output from a channel that is

-- associated with it.  The captured packets are placed

-- in entries in the captureBufferTable.  These entries are

-- associated with the dBufferControlEntry on whose behalf they

-- were stored.

dBufferControlTable OBJECT-TYPE

wrapWhenFull(2)    -- FIFO

dCaptureBufferTable OBJECT-TYPE

-- The Statistics Buffer Group

-- The Statistics Buffer group monitors variables like the

-- Alarm group but instead of checking thresholds, it logs

-- entries and calculates a moving average and standard

-- deviation.

statBufferControlTable OBJECT-TYPE

wrapWhenFull(2)    -- FIFO

statBufferTable OBJECT-TYPE

-- The Reachability group

-- The Reachability group measures the round-trip times

-- of ICMP (ping) and UDP echo's to a specific IP address.

-- It also calculates a reachability estimate based on the

-- formula:

-- R = 1 - ICMPWeight*(TotalICMPTime/ReceivedICMPEchos) -

--	     UDPWeight*(TotalUDPTime/ReceivedUDPEchos)

reachabilityTable OBJECT-TYPE

-- The Discovery group

-- The Discovery group records network accesspoints and

-- connections on several levels of the network layer stack.

-- Recorded are Ethernet, IP, UDP and TCP information.

-- This group is comparable to the RMON host and matrix groups.

discNodeControlTable OBJECT-TYPE

discEthernetNodeTable OBJECT-TYPE

discIPNodeTable OBJECT-TYPE

discUDPNodeTable OBJECT-TYPE

discTCPNodeTable OBJECT-TYPE

discEthernetNodeTimeTable OBJECT-TYPE

discIPNodeTimeTable OBJECT-TYPE

discUDPNodeTimeTable OBJECT-TYPE

discTCPNodeTimeTable OBJECT-TYPE

discConnControlTable OBJECT-TYPE

discEthernetConnSDTable OBJECT-TYPE

discIPConnSDTable OBJECT-TYPE

discUDPConnSDTable OBJECT-TYPE

discTCPConnSDTable OBJECT-TYPE

discEthernetConnDSTable OBJECT-TYPE

discIPConnDSTable OBJECT-TYPE

discUDPConnDSTable OBJECT-TYPE

discTCPConnDSTable OBJECT-TYPE