# Ressources

The ARC88 reserves six ressources for itself. Four of them take care of the functionality of the ARC88 – the remaining two are responsible for the PnP.

After configuring the ARC 88 through a PnP operating system, the ressources are in the appropriate structures:

| Ressource | Size | Function |
| --- | --- | --- |
| IRQ | - | Data transfer coordination |
| I/O | 80h | PnP |
| 1. memory section | 80h | PnP (PLX PCI Controller) |
| 2. memory section | 20000h | ## RAM |
| 3. memory section | 400h | AD1845‘s |
| 4. memory section | 200h | FPGA |

# RAM

Through the 2. memory section the RAM of the ARC88 can be reached.. This is necessary for the exchange of the audio data and is structured as follows:

Address

Offset	Group	Function	Channels	Sample order

1FFFFh

unused	.

Record	unused	.

unused	.

18000h	Left + Right	.

17FFFh	digital	unused	Sample Right High Word

Play	unused	Sample Right Low Word

unused	Sample Left High Word

10000h	Left + Right	Sample Left Low Word

0FFFFh	7-8	.

Record	5-6	.

3-4	.

08000h	1-2	.

07FFFh	analog	7-8	Sample Right High Byte

Play	5-6	Sample Right Low Byte

3-4	Sample Left High Byte

1-2	Sample Left Low Byte

00000h

By reading of the appropriate memory section record data will be picked up.
By writing of the appropriate memory section play data will be charged to the ARC88.

The access to all of the memory sections has to be done with a 32Bit access only!
Notice, while writing on the memory sections, there are permitted a max. of 16 write accesses only! After that a read access has to be done to any memory address. Whit that the PLX PCI Controler will release the local bus of the ARC 88. By disregarding disturbances and interruptions may occurr.

# AD1845

Through the 3. Memory section the ports of the four AD1845 can be reached.

The adressoffset for this memory section and coherent functions are calculated as follows:

| Bit | Function |
| --- | --- |
| 0..1 | Has to be 0 |
| 2 | 0 = Port of the register number can be reached 1 = Port of the register data can be reached |
| 3 | Has to be 0 |
| 4 | Chip Select ADDA#1 (Chn 1-2) 0 = selected 1 = not selected |
| 5 | Chip Select ADDA#2 (Chn 3-4) 0 = selected 1 = not selected |
| 6 | Chip Select ADDA#3 (Chn 5-6) 0 = selected 1 = not selected |
| 7 | Chip Select ADDA#4 (Chn 7-8) 0 = selected 1 = not selected |
| 8..9 | Has to be 0 |

The access to all of the memory sections has to be done by a 32Bit access!
The AD1845 has 8Bit registers only. The meaning of Bytes of the written/read DWORD in detail (order of the Bytes from low to high):

- Byte: ADDA#1

- Byte: ADDA#2

- Byte: ADDA#3

- Byte: ADDA#4

The appropriate Byte gets to ADDA#n only if the appropriate Chip-Select-Bit at the adress is activated as well.

The access to one of the AD1845 registers is done by writing the registers to the appropriate Port (Bit 2 = 0). After that reading and writing at the selected register will be possible.(Bit 2 = 1).

For the function of the individual registers see the documentation of Analog Devices of AD1845.

# FPGA

Through the 4. Memory section the FPGA of the ARC88 can be reached. It will take care of the entire control of the ARC88. Only 8 Bytes (2 DWORD) of this memory section are used:

- DWORD: Dataport for the Initialization of the FPGA
At the FPGA a valid plotdesign has to be uploaded, so that the ARC 88 will do another function besides the PnP. This happens through a bit-serial Datastream at D0 of the DWORD. The plot files and the appropriate samplesourcecode are available at marian.

- DWORD: Controlport

| Bits of the ControlPort | Function if read access |
| --- | --- |
| 0..7 | Current record offset of the Digital interface. These Bits represent Bits 5..12 of the Byteoffset to the base address of Digital Record. This is the current record position of the Digital interface. |
| 8..15 | current play offset of thedes Digital interface. These Bits represent Bits 5..12 of the Byteoffset to the base addresse of Digital Play. This is the current play position of the Digital interface |
| 16-23 | current record offset of the Analog interface. These Bits represent Bits 5..12 of the Byteoffset to the Base address of ALL analog record channels. This is the current record position of the analog channels |
| 24-31 | current Play offset of the Analoginterface These Bits represent Bits 5..12 of the Byteoffsets to the base address of ALL analog play channels. This is the current play position of the analog channels |

| Bits des ControlPorts | Function if write access |
| --- | --- |
| 0..1 | Samplerate of the digital play interface: 00 = Slave of the dig. Record interface (not implemented) 01 = 48000 Hz 10 = 44100 Hz 11 = 32000 Hz |
| 2 | Digital THRU: (not implemented) 0 = Dig. Input signal will NOT be given out at the dig. output. 1 = Dig. Input signal will be given out at the dig. Output |
| 3 | Reset of the Interrupt request of the card: Through a H-L flank of this Bit the Interrupt registration of the ARC88 will be acknowleged – Additional Interrupts will be possible – Only then (!) |

| Bits des ControlPorts | Function if write access |
| --- | --- |
| 4 | Dig. Record Enable: 0 = Dig. Record disabled 1 = Dig. Record Start AND Reset of the current record position at 0 (Bits 0..7 if readaccess to control port) |
| 5 | Dig. Play Enable: 0 = Dig. Play disabled 1 = Dig. Play Start AND Reset of the current play position at 0 (Bits 8..15 if read access to control port) |
| 6 | Analog Record Enable: 0 = Analog Record disabled 1 = Analog Record Start AND Reset of the current record position at 0 (Bits 16..23 if readaccess to control port) ALL AD1845 have to be enabled through the appropriate register for recording – Otherwise the recording will not be done! |
| 7 | Analog Play Enable: 0 = Analog Play disabled 1 = Analog Play Start AND Reset of the current play position at 0 (Bits 24..31 if read access to control port) ALL AD1845 have to be enabled through the appropriate register for playing – Otherwise the playing will not be done! |

# Remarks

Many analog functions of the ARC88 (Gaining, Monitoring, Routing, Volume, Pitch, Start, Stop, Samplerate, Sampleresolution) can be reached through the registers of the AD1845 exclusivly. For further informations please see the documentation of Analog Devices.

At the moment digital “Record while Play” is impossible, because the ressources of the FPGA are exhausted. If “Digital Record” is necessary, the appropriate plot design has to be uploaded to the FPGA firstly. “Digital Play”requires the same. Depending on which plot design is activated, the plot design for the respective opposite function has to be downloaded.